ECEN 610: MIXED SIGNAL INTERFACE HW.1

Name: Yu-Hao Chen  UIN: 435009528  Section: 601

Instructor: Sebastian Hoyos

TA: Haotian

- Decimation is the process of reducing the sampling rate Fs of a digital signal.

- When a signal is downsampled (Decimation), its frequency components may exceed the new Nyquist frequency:

F’nyquist= Fs/2M |M=decimate factor

This causes high-frequency components to fold back (alias) into the Nyquist range, resulting in aliasing.

FsD= Fs/M

Nyquist D= FsD/2

Ex: Fs=10k Fin:3k and 6k Nyquist BW= 5k

3k no alias, but 6k alias back to 4k

After decimation M=2 FsD=5k Nyquist BW= 2.5k

Now 3k alias back to 2k, and (6k alias to 4k) and then back to 1k

Original:

Decimate:

- Bin: When we perform Fast Fourier Transform (FFT) on the signal, the frequency axis will be divided into N bins, each bin represents a fixed frequency range.

Ex: Fs=10k Hz FFT N :1024

Bin: Δf= 10k/1024= 9.77Hz

Bin1: 0Hz Bin2: 9.77Hz Bin3: 19.54Hz …Bin1024

- FFT can only resolve discrete frequency points. These frequency points are determined by FFT bins(No matter before decimating or after). The bin intervals are:

Bin size= Fs/N

If alias frequency fa does not fall exactly on a certain bin, then it will have a frequency error in the FFT spectrum.

Therefore, we use:

Fa= round(Fa/Bin size) * Bin size

Align to the nearest FFT bin so that the FFT can parse it correctly.

Ex: Fs: 1000Hz, FFT N: 16, M=2 (FsD=500Hz ND=8), Fin:1200Hz

Original Fa: |1200- 1*1000|= 200Hz

FFT Bin: Fs/N= 62.5Hz

Normalized Fa: around(Fa/Bin size) *bin size= 187.5Hz (Bin3)

FaD= Fa mod FsD= 200 mod 500= 200Hz

Still need to normalized again.

- Math for doing this

- Fa=|Fin-kfs|, k=round(fin/fs)

- FaD= Fa mod FsD

- Normalized Bin: Bin=round(Fa/Bin size) * Bin size (for both Fa FaD)

(Fa normalize and FaD normalized should be the same)

- After downclocking, the FFT Bin spacing (frequency resolution) remains unchanged.

- After downclocking, the number of FFT Bins is reduced, but the frequency point spacing remains the same.

- If alias frequency Fa corresponding bin remains unchanged, it will still fall into the same frequency bin after downscaling to ensure correct alignment.

- Simulation

Fs: 1000Hz, Fin: 1200Hz, FFT N:882, M=2

Estimation: fa=|fin-kfs|=200, k=round(fin/fs)=1

Bin size: Fs/N= 1000/822= 1.13379Hz

Normalized Fa= round(Fs/ Bin size) *Bin sized= 199.5468Hz

Code for Fa and normalized

FFT and pre-plotting

Plotting